The King's College - 2026

Year 12 Computer Science General

# Task 8 UniFi Design Center Guide

Use UniFi Design Center to create the visual evidence for your high value domestic network design.

## What UniFi Design Center Is Used For

UniFi Design Center helps you plan a network before installation. For this task, use it to:

import or recreate the floorplan

place network devices

plan wireless access point locations

estimate wireless coverage

show camera or outdoor coverage where relevant

produce screenshots or exports for your submission

## Step 1: Prepare The Floorplan

Select a house listing with a clear floorplan.

Save or screenshot the floorplan for classroom assessment use.

Record the listing URL and date accessed.

If the floorplan includes a scale or room dimensions, use them to estimate size.

If scale is unclear, state that your scale is approximate.

## Step 2: Start The UniFi Project

Open UniFi Design Center.

Create a new project.

Import the floorplan image or recreate the property layout.

Set the scale as accurately as possible.

Add wall materials or wall assumptions where the tool allows.

## Step 3: Place The Network Core

Choose a sensible network core location, such as:

garage data cabinet

utility room

study cupboard

central hallway cupboard

communications cupboard

Place or label:

modem or ONT

gateway/router

firewall function or security gateway

switch or PoE switch

internet/WAN entry point

Build in this order:

label the internet/WAN entry point

place the modem or ONT close to the WAN entry point

place the gateway/router/firewall after the modem or ONT

place the switch or PoE switch at the network core

add ethernet or PoE links from the switch

place access points to solve coverage needs

add fixed wired devices where appropriate

explain wireless networks or SSIDs such as family, guest, IoT, camera or home office

This order matters because the design needs to show the network structure, not just Wi-Fi coverage.

| Item | What it is | Why it belongs in the design |
| --- | --- | --- |
| Internet/WAN entry | The provider/internet side outside the home LAN. | Shows the difference between LAN and WAN. |
| Modem/ONT | Connects the home to the provider service. | Represents the physical internet connection before local routing. |
| Gateway/router | Routes traffic between LAN and WAN. | Lets local devices communicate with external networks. |
| Firewall function | Filters traffic according to rules. | Protects the home network and supports security decisions. |
| Switch or PoE switch | Central wired LAN device. | Forms the centre of the star topology and can power APs/cameras. |
| Ethernet/PoE links | Wired twisted pair links. | Gives reliable connections for APs and fixed high-demand devices. |
| Wireless access points | Provide Wi-Fi access to the wired LAN. | Supports mobile devices and whole-home coverage. |
| Network groups/SSIDs | Family, guest, IoT, camera or home office networks. | Shows authentication, encryption, firewall and isolation decisions. |

## Step 4: Place Access Points

Place wireless access points to support:

living areas

study or home office

bedrooms

outdoor entertaining areas where relevant

high-use media or gaming spaces

Avoid placing every access point in one corner of the house. Use the floorplan and coverage evidence to justify placement.

For each access point, write a reason:

This access point is placed near ______ because ______.

Example:

This access point is placed near the upstairs bedrooms because the heatmap shows weak coverage from the ground-floor access point and students may use laptops or phones in those rooms.

## Step 5: Plan Wired Links

Use wired ethernet for fixed or high-demand devices where suitable:

access points

cameras

home office desk

TV/media area

gaming area

NAS or home server

printer

Label these as ethernet, twisted pair or PoE where relevant.

For each wired link, write a reason:

This device uses ethernet because ______.

Example:

The home office dock uses ethernet because video calls and file access need a stable connection and the device stays in one place.

## Step 6: Screenshot Required Evidence

Your submission should include:

source floorplan or property source record

UniFi floorplan with devices placed

Wi-Fi coverage or heatmap evidence

device list or visible device labels

any camera/outdoor coverage evidence if used

logical topology section showing how the devices connect

## Step 7: Check Before Submission

Before you submit, check:

every major device shown in UniFi is explained in your written justification

every major written recommendation appears in the design evidence

access points are placed for coverage, not decoration

wired links are used for fixed high-demand devices

guest/IoT/private network separation is explained

limitations are stated if scale or wall materials were estimated

## Common UniFi Design Mistakes

| Mistake | Fix |
| --- | --- |
| Using a floorplan screenshot without any network design evidence | Import or recreate the floorplan and add network devices. |
| Placing one router and no access points in a large house | Use multiple access points where coverage requires it. |
| Ignoring wall materials or house size | State assumptions and use coverage evidence to support placement. |
| Showing only Wi-Fi coverage and no logical topology | Add a topology section explaining WAN, LAN, switch, APs and clients. |
| Adding premium devices with no reason | Link every device to a floorplan need, user need, reliability need or security need. |